# Tonick Watering Written Specifications

# Part 1 – General Arrangement

- The irrigation control system shall feature a two-wire translator module for control of up to 127 standard 24VAC solenoid valves over a single pair of wires. The two-wire translator module shall be DIN rail mountable and may co-exist with “conventional” output modules featuring individual wires per solenoid valves. The two-wire translator module shall connect to the host controller via a serial MODbus interface, or digital input module. The two-wire translator module shall have an independent power supply and be grounded separately from the other equipment in the control cabinet.

# Part 2 - Two-Wire Translator Module

- The two-wire translator module shall be manufactured by the same manufacturer as the decoders and shall carry all the same necessary approvals and compliance.

  - Translator Module Characteristics

    - The output of the two-wire translator module shall not exceed 32VAC (nominal 24VAC~) at 50 or 60 Hz, suitable for direct-burial wiring in landscape irrigation applications.

    - The two-wire translator module shall include two separate, clearly labeled, heavy-duty two-wire path screw terminals for wiring in different directions.

    - The two-wire translator module shall also permit operation of a Pump/Master Valve output via decoder in the two-wire path, if desired.

    - The two-wire translator module shall also permit operation of Sensor inputs via decoder in the two-wire path, if desired.

    - The two-wire output module shall also include an LED light for programming and diagnostic purposes.

    - The two-wire translator module shall operate dedicated, programmable field decoders, manufactured by the same manufacturer as the two-wire translator.

    - The two-wire translator module must be capable of operating multiple field decoders, plus a decoder-operated Pump/Master Valve output, simultaneously.

# Part 3 - Field Decoders

- Decoder Characteristics

  - The field decoders shall be completely waterproof, rated at a minimum of IP68 for total immersion.

    - Each field decoder shall be programmable/reprogrammable with the desired station address and shall not require separate serial numbers of any kind.

    - Each field decoder must include a waterproof status LED light to confirm programming and operations, and serve as a field diagnostic aid.

  - The field decoders shall be tropical strength and rated for use in harsh environments.

  - The programmable/reprogrammable field decoders shall receive their station addresses from a programming tool supplied by the decoder manufacturer. The installer shall select the station number to program into the field decoder, prior to installation in the field at valve locations.

  - The field decoders should be capable of operating on the same line as combo decoder/actuator devices and sensor decoders.

  - The field decoder output to the solenoid shall be 19 -32 VAC~, 50/60 Hz, and the output of an active station shall be measurable on a standard voltmeter.

    - The wiring distance from the decoder to the solenoid shall be limited only by the total distance from the controller to the solenoid and the wire diameter, as indicated in the manufacturer’s wiring table.

# Part 4 - Field Wiring (two-wire path)

- The field decoders shall have color-coded wires to facilitate proper connection to the two-wire path and to the valve solenoid. There shall be no polarity on the two-wire path, which means it does not matter which path wire is connected to which incoming lead, and there shall be no polarity on the output wires to the 24VAC solenoid.

  - The two-wire path should be sized in accordance with the decoder manufacturers recommendations and only irrigation specific cable designed for two-wire applications should be used.

  - Wiring between the two-wire output module and the field decoders shall consist of direct-burial-rated standard irrigation wiring of an appropriate diameter for the maximum distances per the manufacturer’s specifications.

  - All field decoder connections shall be made with gel-filled waterproof splice connectors designed for standard in-ground landscape irrigation purposes.

  - It is permissible, but not required, to run field wiring in plastic or metal conduit for resistance to pests.

# Part 5 - Earth Grounding

- The two-wire translator module shall be thoroughly earth grounded in accordance with the manufacturer’s specifications, ideally to a resistance of 10 ohms or less, and should be separately grounded to any other equipment in the control cabinetry.

- The two-wire path does not require additional earth grounding or surge suppressors of any kind in most installations.

- A Line Terminating Unit shall be installed each end of the two-wire path to complete the pathway and can be paired with decoders at that point.